# Motor mounting bracket - static and modal check

Example study / L-bracket, Al 6061-T6, 6 mm plate
Analyst: Claude via Ansys connector    Date: 2026-10-25    Rev: A    Units: mm, N, MPa, s
Verdict: REVIEW

The bracket carries the 500 N shock load with a safety factor of about 2.0 on yield and 0.81 mm deflection. With a realistic motor rotary inertia, mode 2 sits at 131 Hz, 9.5 % above twice running speed (120 Hz), inside the 20 % resonance band. Stiffening or a bump test is recommended before release.

# 1. Scope

Linear static check under a 500 N shock load plus gravity, and modal check against 3600 rpm.

# 2. Model

L-bracket: 80 mm base with two 8.5 mm bolt holes at X = 20 and 60 mm, 60 mm upright, 40 mm wide, 6 mm thick.

## Material

| Property | Value | Unit | Note |
| --- | --- | --- | --- |
| Young's modulus | 68900 | MPa |  |
| Poisson's ratio | 0.33 |  |  |
| Density | 2.70e-9 | t/mm^3 |  |
| Yield strength | 276 | MPa |  |

## Mesh

| Item | Value | Unit | Note |
| --- | --- | --- | --- |
| Element | SOLID187 (10-node tet) |  |  |
| Global / local size | 2 / 1 | mm |  |
| Nodes | 84k |  |  |

## Supports

Both bolt bores fully fixed; base underside free

## Loads

500 N in +X on the motor (RBE3 to 20x20 mm patch)

Gravity -Y on bracket and 1.2 kg motor

# 3. Results

| Quantity | Value | Unit | Location | Limit | Margin | Status |
| --- | --- | --- | --- | --- | --- | --- |
| Max von Mises (outside singular zones) | 135 | MPa | Base top, between corner and bolt 1 | 276 MPa yield | SF 2.0 | PASS |
| Max deflection | 0.81 | mm | Top of upright |  |  |  |
| Mode 1 | 76.5 | Hz |  | outside 48-72 / 96-144 Hz | +6 % above 1x band | WARN |
| Mode 2 | 131.4 | Hz |  | outside 96-144 Hz | +9.5 % from 2x | FAIL |

Von Mises stress, 500 N + gravity

Mode 2 (point-mass run)

# 4. Verification

| Check | Expected | Actual | Status | Note |
| --- | --- | --- | --- | --- |
| Equilibrium, X | 500 N | 500.0 N | PASS |  |
| Equilibrium, Y (weight) | 12.6 N | 12.6 N | PASS |  |
| Hand calc, upright root bending | ~85 MPa | 78 MPa | PASS |  |
| Bolt-bore peak | converged | 203 -> 300 MPa, rising | WARN | Singularity at fixed bore; excluded |

## Mesh convergence: max von Mises outside singular zones

| Mesh | Nodes | Value | Change |
| --- | --- | --- | --- |
| coarse | 15k | 131.5 MPa |  |
| 1 mm local | 87k | 135.5 MPa | 3.0 % |
| 0.75 mm local | 84k | 136.2 MPa | 0.5 % |

# 5. Assumptions

Motor rotary inertia 1 t*mm^2 (80 mm diameter motor)

Bolt bores fully fixed

# 6. Limitations

Stresses at the bolt holes are not meaningful with fixed bores

Shock treated as static; pulses shorter than ~11 ms may amplify up to 2x

# 7. Conclusions

Static strength adequate (SF 2.0)

Mode 2 too close to 2x running speed

# 8. Recommendations

Add a corner gusset or thicken the upright, then re-check mode 2

Bump-test the prototype

# 9. Files

| File | Path |  |  |
| --- | --- | --- | --- |
| APDL model | examples/motor-bracket/bracket.mac |  |  |

Prepared: Claude via Ansys connector

Checked by: ____________________   Date: ________

Approved by: ___________________   Date: ________

Generated with the Ansys connector for Claude (unofficial). Results depend on the model, inputs and assumptions above and must be reviewed by a qualified engineer before use.